Tribute Band Technical & Hospitality Rider

(Large Venue Edition)

Band Name: [Your Tribute Band Name]
Tribute To: [Original Artist or Band]
Contact: [Band Manager or Tour Contact Info]
Date of Show: [Insert Date]
Venue: [Insert Venue Name + Location]

TECHNICAL REQUIREMENTS

Front-of-House PA

- Professionally maintained 3- or 4-way PA system appropriate to venue capacity

- Capable of 110 dB SPL at FOH without distortion

- Midas, Allen & Heath, or Avid console preferred

- Qualified FOH engineer provided by venue unless otherwise arranged

- Clear sightlines from FOH to stage

Monitors / In-Ear Monitoring

- 4–6 independent monitor mixes (wedges or IEMs)

- Sidefills if stage size exceeds 24' wide

- 1 talkback mic routed to monitors only

Inputs & Mics (Sample Input List)

| Channel | Source | Mic/DI Preferred |
| --- | --- | --- |
| 1 | Kick | Shure Beta 52 / D6 |
| 2 | Snare Top | SM57 |
| 3 | Snare Bottom | SM57 |
| 4 | Hi-Hat | SM81 / C414 |
| 5 | Tom 1 | e604 |
| 6 | Tom 2 | e604 |
| 7 | Floor Tom | e604 / D4 |
| 8 | Overhead L | C214 / SM81 |
| 9 | Overhead R | C214 / SM81 |
| 10 | Bass DI | Radial / Countryman |
| 11 | Guitar L | SM57 / e906 |
| 12 | Guitar R | SM57 / e906 |
| 13 | Keys L | DI |
| 14 | Keys R | DI |
| 15 | Lead Vocals | SM58 / Beta 58 |
| 16+ | Backing Vocals | SM58 / Beta 58 |

Add inputs for tracks, horns, extra percussion, or dual keys as needed.

LIGHTING

- Full wash coverage with front light, backlight, and side fill

- Minimum 4 LED color washes + 2–4 spot/follow lights

- If moving lights are available: chase cues, dynamic transitions, and strobes optional

- Hazer OK (preferred for certain tribute acts)

BACKLINE (If Provided by Venue or Festival)

- Drums: 5-piece professional kit (DW, Yamaha, Pearl) + hardware & cymbals

- Guitar Amps: Fender Twin, Vox AC30, Marshall DSL/800 series

- Bass Amp: Ampeg SVT, GK 800RB, or comparable

- Keyboards: Nord, Roland, or Yamaha stage piano (88 keys preferred)

- Keyboard stands, sustain pedal, DI boxes, guitar stands

STAGE & POWER

- Minimum stage size: 24’ x 16’

- Clean, grounded AC power: 4–6 drops across stage (20A minimum)

- Risers: Optional but preferred for drums (8’x8’) and keys (6’x6’)

LOAD-IN / SOUND CHECK

- Load-in: Minimum 3 hours prior to doors

- Soundcheck: Minimum 60–90 minutes, line check allowed if needed

- Crew Access: Early entry for techs/stage manager if available

- Parking: Access for 1–2 vehicles or trailer near stage loading dock

HOSPITALITY

Dressing Room: Clean, secure, and climate-controlled with chairs, mirrors, table
Catering / Hospitality (For 5–7 members):

- Bottled water, coffee, tea, soft drinks

- Healthy snacks (fruit, protein bars, veggie trays)

- Sandwich or hot meal (negotiable) if on-site for 4+ hours

- Hand towels, access to restroom or shower if available
Buyout: If food is not available, a $15–$20 per band member buyout is acceptable

COMPENSATION & SETTLEMENT

- Payment due night of performance, unless otherwise agreed

- Acceptable forms: Check, Cashier’s Check, Zelle, PayPal, or Wire

- Any applicable deposit to be paid in advance by agreed date

- Merchandise split: 80/20 in favor of artist unless otherwise noted

- Venue must supply 2 tables + lighting for merch sales, if applicable

MARKETING & PROMOTION

- Band to supply high-res promo assets, logo, and EPK upon booking

- Venue to include act in all online, print, and event listings

- Social media co-promotion encouraged (Facebook, Instagram, NTBA platform)

INSURANCE

- COI available upon request

- Venue should hold liability coverage for on-site injury or property damage

CANCELLATION POLICY

- If canceled within 30 days of show date, 50% of agreed fee is due

- If canceled within 14 days, 100% of fee is due unless rescheduled